LATERAL LOAD PATH

QA/QC REVIEW REPORT

[PROJECT NAME]

[Project No. XXXXX]

[Drawing set title / issue date / revision reviewed; calcs and geotech report reviewed]

Prepared by: [Name]

Date: [Date]

Scope: Lateral (Seismic) Load Path and Design Basis (V3)

This report traces the lateral load path and checks the stated seismic design basis and detailing qualitatively against ASCE 7-22 and the referenced material standards. It does not include load-magnitude or capacity calculations, and it does not replace the engineer of record's responsibility or plan check.

# Executive Summary

This report documents a lateral (seismic) load path trace of the structural drawing set identified above. It identifies the seismic force-resisting system in each direction, checks the stated design basis against ASCE 7-22, traces a representative line of resistance in each direction link by link from the diaphragm to the soil, and individually traces every condition that differs from the typical line in a way that could affect the lateral load path.

[Summarize: SFRS in each direction and Table 12.2-1 row; SDC; diaphragm types; number of lines of resistance reviewed and number traced individually; headline findings grouped by type: broken/unconfirmed path, design-basis inconsistency, detailing non-conformance, reference gap.]

Example: "The building uses light-frame wood shear walls (B.23) in both directions, SDC D. Of 9 lines of resistance, 6 matched the typical condition. Three conditions were traced individually. Four findings: one design-basis inconsistency (Ω₀), two broken or unconfirmed load path links (open-front diaphragm analysis; hold-down continuity at SW-6), and one reference gap (wind basis)."

# Scope & Limitations

This review covers the lateral load path, the stated seismic design basis, and system detailing, qualitatively. It does not cover:

- Load magnitude, member sizing, or capacity checks. The review confirms the basis is stated and consistent, not that the numbers are right.

- Wind load determination (ASCE 7-22 Chapters 26-31), unless noted. Where wind governs, the path is traced but the wind basis is not independently reviewed.

- The gravity load path (see the gravity-load-path-qaqc review, if available).

- Systems or conditions not covered by this skill's reference material. These are flagged individually as reference gaps.

Code basis: ASCE 7-22; ACI 318-19. Items tied to SDPWS-21, AISC 341/358, TMS 402, or IBC/CBC that are marked "verify" in the skill references are written as confirmation requests, not as findings. [List any sheets, calcs, or reports that were not available and how that limited the review.]

# System Identification & Design Basis

- Lateral system, N-S: [system, Table 12.2-1 row, detailing level]

- Lateral system, E-W: [system, Table 12.2-1 row, detailing level]

- Diaphragms by level: [material; classification used in calcs]

- Foundation: [type; geotech report reference and seismic items]

- Supported heavy components / nonbuilding structures: [list]

## Design Basis: Stated vs. Expected

| Item | Stated on Drawings / Calcs | Expected per ASCE 7-22 | Status |
| --- | --- | --- | --- |
| Risk Category / I_e | [e.g. II / 1.0] | Per IBC Table 1604.5 and ASCE 7-22 Table 1.5-2 | [OK / Flag] |
| Site Class; S_DS, S_D1 | [e.g. D (default); 1.12, 0.64] | Geodatabase values for the stated site class; default site per §11.4.3 | [OK / Flag] |
| SDC | [e.g. D] | Tables 11.6-1/11.6-2; S1 ≥ 0.75 override (E/F) | [OK / Flag] |
| SFRS (each direction) and Table 12.2-1 row | [e.g. N-S and E-W: B.23 light-frame wood WSP walls] | Permitted in the SDC; within the height limit | [OK / Flag] |
| R / Ω₀ / C_d | [e.g. 7 / 2.5 / 4.5] | All three from the same row; flexible-diaphragm Ω₀ reduction applied correctly | [OK / Flag] |
| ρ (each direction) | [e.g. 1.0 / 1.3] | §12.3.4 conditions shown | [OK / Flag] |
| Irregularities | [e.g. Type 1a torsional, Type 4 out-of-plane offset at Level 2] | Consequences applied (3D model, Ω₀ on supporting elements, 25% increase) | [OK / Flag] |
| Diaphragm classification | [e.g. roof: flexible; Level 2: semirigid envelope] | §12.3.1; semirigid default; open-front never flexible | [OK / Flag] |
| Analysis procedure | [e.g. ELF] | Permitted per Table 12.6-1 | [OK / Flag] |
| Base shear / C_s | [e.g. 0.175W] | Within C_s limits; matches calcs | [OK / Flag] |
| Drift limit | [e.g. 0.020h_sx] | Table 12.12-1 for RC and system | [OK / Flag] |

# Load Path Trace

## Typical Line of Resistance, [Direction]: Full Trace

Representative condition: [e.g. "Shear wall lines 2, 4, 6 (N-S), typical interior wall SW-6"]. Traced in full below. Every other line in this direction was compared against it. Repeat this section for the other principal direction.

| Link | Element | Mechanism / Force Direction | Verified? | Notes / Flags |
| --- | --- | --- | --- | --- |
| 1. Load into diaphragm | Roof WSP sheathing; CMU parapet wall out-of-plane | Wall anchorage strap to continuous crosstie (subdiaphragm), per 5/S-501 | Yes | Anchorage force basis F_p with k_a stated in calcs? [confirm] |
| 2. Across diaphragm / chords | 15/32" WSP, blocked, 10d @ 4" boundary; double top plate chord | Edge nailing per diaphragm schedule; top-plate splices with strap | Yes | Chord splice detail 8/S-502 shown at every splice location. |
| 3. Into vertical element | Drag strut along Grid 3 (wall shorter than diaphragm depth) | Strap-spliced top plates, sheathing edge-nailed into strut | Verify | Collector force level not stated (light-frame wood exception may apply, §12.10.2.1). |
| 4. Down the element (each story) | Shear wall SW-6 Levels 2 and 1 | Sheathing nailing per shear wall schedule; floor-to-floor via rim and blocking nailing and sill shear nailing | Yes | Stacks over SW-6 below; no offset. |
| 5. Overturning path | Hold-down HD-1 at each end; 4x posts | Floor-to-floor strap at Level 2; HD anchor to footing | No | Hold-down at the north end of Level 2 lands on the floor beam, not on a post below. See Finding 3. |
| 6. Base anchorage | Sill plate; 5/8" anchor bolts @ 32" with 3"x3" plate washers | Anchor bolts in shear; HD anchor in tension | Yes | Per 3/S-401. |
| 7. Foundation / soil | Continuous footing F-2 | Overturning by footing and soil bearing; sliding by friction and passive per geotech | Verify | Geotech friction and passive values cited in calcs? [confirm] |

## Atypical Conditions: Individually Traced

The following conditions differed from the typical line in a way that could affect the lateral load path and were traced individually. Lines not listed matched the typical condition.

| Condition / Location | Path | Mechanism / Required Basis | Verified? | Notes / Flags |
| --- | --- | --- | --- | --- |
| Grid C Level 2: discontinuous wall (out-of-plane offset, Type 4) | Wall above supported on beam B-12 and posts | Beam and posts designed with Ω₀ (§12.3.3.4)? | Verify | Calcs not provided for B-12; flag. |
| Grid 1: open-front line (no walls) | Diaphragm cantilevers to Grid 2 | Diaphragm analyzed as semirigid / rigid; torsion distributed to cross walls | No | Calcs treat roof as flexible at the open front. See Finding 2. |
| Rooftop unit RTU-1 (2,500 lb) | Curb to roof framing | Anchorage to curb and curb to framing for F_p (Ch. 13) | Verify | Anchorage detail references the manufacturer; no F_p basis shown. |

# Findings

[Number every finding and tag its type. Rank by impact on the load path: broken or unconfirmed links first, then design-basis inconsistencies, detailing non-conformances, and reference gaps. Give the location, sheet reference, governing code section, and a recommended action (RFI, revise, provide calc, field verify).]

| No. | Type | Location / Sheet | Finding | Code Reference | Recommended Action |
| --- | --- | --- | --- | --- | --- |
| 1 | Design-basis inconsistency | General notes, S-001 | Ω₀ listed as 3.0 with system B.23 (Ω₀ = 2.5). | ASCE 7-22 Table 12.2-1 | Correct Ω₀; confirm collector and anchorage forces used the right value. |
| 2 | Broken / unconfirmed path | Roof, Grid 1 open front | Open-front diaphragm analyzed as flexible. | §12.3.1 (project standard: never flexible) | Reanalyze as semirigid or rigid, including torsion to the cross walls. |
| 3 | Broken / unconfirmed path | SW-6 north end, Level 2 to Level 1 | Hold-down tension path not continuous to the foundation. | Overturning load path | Provide a post and strap or rod continuous to the footing, or revise the wall. |
| 4 | Reference gap | Building-wide | Wind governs E-W base shear; wind basis not reviewed by this skill. | ASCE 7-22 Ch. 26-28 | Separate wind review. |

# Appendix

## Details and Schedules Relied On

[List the project-specific details and schedules used for each link, and any typical or standard detail relied on where no project-specific one exists.]

## Items to Confirm Against Material Standards

[List the "verify" items: requirements the skill's references draw from SDPWS-21, AISC 341/358, TMS 402, NDS, or IBC/CBC that should be confirmed against the governing document before being treated as findings.]